WDT 16-CH DVR Card - Tips for installation

- Mother board with Intel Chipset, CPU PIII 800Mhz & up, ATI graphic card with 32MB RAM

- Using Windows 2000 or XP OS

- Partition HD into at least two, i.e. “C” and “D”.

- Install the software without the DVR card

- Put the DVR card into the PC, use the PCI slot closer to the CPU

- Install the device drivers. After finish installation, you should have total 8 - 4 audio and 4 video device drivers named “Progressive DVR Audio…” and “Progressive DVR Video….” under “Sound, video and game controllers” of the “Device Manager”. Reboot the PC.

- Start the program, click OK to the password prompt window

- Go to “Setup” -> “Device”, enable the camera channels by checking off the box next to the camera list. You should see the camera activities if it is connected

- Go to “Calendar” to change the mode from recording to display or others

- Remote view through IE: Type in the IP address of the DVR server (PC with the DVR card installed). Notice that you can’t do remote view if your server is connected to the LAN using the router.

- Install the program to the remote PC, click the icon “DVR network System”. You can have the full features like using the server. Notice that first you need to add the user and define the privileges under “Setup” of the server. Do not have a user having all the privileges like the administrator.